# Extraction Booth Procedure

Standard Operating Procedure

Title: Extraction Booth Procedure

## 1. Purpose

This Standard Operating Procedure establishes the standard workflow for operating the extraction booth during primary extraction, flush cycle, and final purge. The purpose of this procedure is to ensure consistency in solvent movement, reactor handling, recovery, filtration, temperature control, and final purging of biomass.

## 2. Scope

This SOP applies to all trained personnel performing extraction booth operations, including reactor vacuuming, solvent charging, soaking, recovery, flush cycle, and final purge activities.

## 3. Responsibilities

Extraction Operators are responsible for completing each step of the extraction process in the correct sequence and documenting required temperatures and process observations.

Supervisors/Managers are responsible for ensuring operators are trained on this SOP and that all required documentation is complete.

Quality and Compliance Personnel are responsible for reviewing records as required and ensuring the process is completed within approved operational parameters.

## 4. Safety and Critical Control Requirements

- Only trained personnel may perform this procedure.

- All required PPE must be worn at all times in the extraction booth.

- Nitrogen and solvent handling must be performed with caution and in accordance with facility safety procedures.

- Operators must verify that all valves, lines, vessels, and connections are secure before beginning the run.

- Critical Temperature Requirement: solvent must be maintained at below -40°F during the flush cycle.

- Required temperature documentation shall include:

  - Picture of the temperature on the solvent chiller

  - Picture of the plate temperature

  - Temperature updates posted to Slack

## 5. Equipment and Materials

- Reactor

- Solvent supply

- Small solvent tank

- Nitrogen line

- Filtration system

- Recovery vessel

- Dewax vessel

- Heat exchangers

- Mixer

- Temperature monitoring devices

- Camera or phone for temperature photos

- Slack access for temperature posting

## 6. Procedure

### 6.1 Primary Extraction

#### Step 1 – Vac Down Reactor

Vacuum down the reactor prior to charging solvent.

#### Step 2 – Charge Solvent

Send 500 lbs of solvent into the reactor.

#### Step 3 – Initial Soak and Agitation

Allow the reactor to soak for 30 minutes.
During that 30-minute soak, run the mixer for 5 minutes to agitate and turn over the biomass.

#### Step 4 – Clear Basket Filter and Pressurize

Use the small solvent tank to burp the bottom of the reactor, clearing the basket filter as effectively as possible.
After clearing the filter, attach the nitrogen line to the top of the reactor and begin pressurizing.

#### Step 5 – Send Gas to Filtration and Recovery

Send gas over to filtration and then into recovery.
As the gas begins to develop a consistent stream and the recovery tank appears to be building, turn the heat exchangers on.

#### Step 6 – Shut Reactor and Heat Exchangers Down When Clear

When the sight glass on the dewax vessel is clear:

- close the nitrogen valve on the reactor,

- close the outlet on the bottom of the reactor, and

- turn off the heat exchangers.

#### Step 7 – Close Inlet and Oily Valves

Close the inlet and oily valves on the stingers.

#### Step 8 – Close Recovery Vessel Inlets

Close the inlets into the recovery vessel and recover as needed for the next run.

### 6.2 Flush Cycle

#### Step 9 – Begin Flush Cycle

Once gas level has been reached for flush (500 lbs), proceed with the flush cycle.

#### Step 10 – Verify Solvent Temperature

Before sending flush solvent, confirm that solvent temperature is below -40°F.

Complete the following documentation:

- take a picture of the temperature on the solvent chiller,

- take a picture of the plate temperature, and

- post temperatures to Slack.

#### Step 11 – Send Flush Solvent

Send 500 lbs of flush solvent to the reactor.
If needed, burp nitrogen from the reactor before sending gas.

#### Step 12 – Flush Soak

Allow the flush to soak for 10 minutes.
Run the mixer for the last 5 minutes of the soak.

#### Step 13 – Burp and Re-Pressurize

Burp the bottom of the reactor with nitrogen and then begin pressurizing.

#### Step 14 – Resume Flow to Recovery

Once the reactor is pressurized, begin sending flush toward recovery.
Open the recovery valves and lower the filtration valves slightly to allow flow to resume.
When the flow picks back up, turn the heat exchangers back on.

#### Decision Point – Has Flow Resumed?

If flow has not resumed, adjust valves as needed until flow resumes.
If flow has resumed, continue with the process and keep heat exchangers on.

### 6.3 Final Purge

#### Step 15 – Push Nitrogen and Begin Burping

Once all gas is gone, push nitrogen through the bottom of the reactor one more time, then begin burping the reactor.

#### Step 16 – Final Burp

Burp for approximately 30 minutes.
The objective is to remove as much remaining gas from the biomass as possible.

#### Decision Point – Is Everything Clear?

Evaluate whether everything is clear, or as clear as it is expected to get.

- If no, continue burping until acceptable clarity is reached.

- If yes, proceed to shutdown steps.

#### Step 17 – Shutdown and Closeout

Once clarity is reached:

- Close off inlets to recovery and begin pumping down filtration.

- Turn off nitrogen on the reactor.

- Close off the inlet to the dewax vessel.

The extraction booth process is then complete.

## 7. Documentation Requirements

The following shall be documented for each run:

- Date and time of run

- Operator name

- Reactor vacuum confirmation

- Solvent charge amount

- Soak times

- Mixer run times

- Flush solvent amount

- Solvent chiller temperature

- Plate temperature

- Confirmation that temperatures were posted to Slack

- Notes on valve adjustments, if needed

- Final burp duration

- Any deviations or issues observed during the run

## 8. Critical Control Points

- Reactor must be properly vacuumed before solvent charge.

- Solvent charge and flush quantities must be verified.

- Solvent temperature during flush must remain below -40°F.

- Heat exchangers must only be turned on and off at the proper points in the process.

- Final burping must continue until the system is as clear as operationally achievable.

## 9. Deviations

Any deviation from this SOP must be documented and reported to management immediately. Any material or process impacted by a deviation must be reviewed before release to the next stage.

## 10. Related Records

- Extraction booth run log

- Temperature log

- Slack temperature post record

- Equipment maintenance log

- Batch production record

# Post - Processing Procedure

Standard Operating Procedure Post Processing

Title: Post-Processing Workflow

## 1. Purpose

This Standard Operating Procedure establishes the workflow for extraction and post-processing activities, including the Pot Pour Path, THCa Path, and HTE Path, to ensure consistency in production, documentation, product routing, and quality control.

## 2. Scope

This SOP applies to all extraction and post-processing personnel involved in biomass extraction, off-gassing, centrifuging, THCa processing, HTE processing, product path decisions, testing, and production queue transfers.

## 3. Responsibilities

Extraction Team is responsible for selecting the extraction run type and completing the extraction process.
Post-Processing Team is responsible for handling off-gassing, centrifuge processing, oven processing, milling, refinement, and routing of materials.
Quality/Testing Team is responsible for sample testing, review of results, and confirming whether material meets internal standards.
Production Management is responsible for final product path decisions and release into the appropriate production queue.

## 4. Definitions

THCa: Tetrahydrocannabinolic acid fraction collected during processing.
HTE: High terpene extract fraction collected during processing.
LD: Liquid Diamonds.
HP: Happy Place.
Pot Pour Path: 100 lb extraction workflow.
Minor Run: 200 lb extraction workflow producing THCa and HTE outputs.

## 5. Procedure

### 5.1 Extraction Run Selection

At the start of processing, the appropriate extraction run shall be selected based on production needs. The two approved pathways are:

- Option 1: 100 lb Pot Pour

- Option 2: 200 lb Minor Run

## 5.2 Option 1: 100 lb Pot Pour Path

When a 100 lb pot pour run is selected, the following steps shall be completed:

- Material shall be placed in the warm off-gas area for 10 days.

- Material shall be stirred once daily during the off-gas period.

- After the off-gas period is complete, the material shall be moved to the post-processing lab.

- The material shall then proceed through the centrifuge process.

- Outputs shall be collected and separated as THCa and HTE.

- Final weights for both THCa and HTE shall be recorded in the applicable batch record.

- Once weights are confirmed and documented, the pot pour path is complete.

## 5.3 Option 2: 200 lb Minor Run

When a 200 lb minor run is selected, the initial outputs shall be identified as THCa + HTE. At that point, the material shall be divided into one of two downstream paths:

- THCa Path

- HTE Path

## 5.4 THCa Path

If material is assigned to the THCa Path, the following steps shall be completed:

- THCa material shall be placed in the THCa oven for 16 hours.

- After oven processing, the THCa shall be milled.

- A product path decision shall then be made for the THCa.

Approved THCa product path options are:

- Sell THCa

- Make LD

- Formulate into badders or sugars

If the THCa is designated for LD or another clarity-sensitive product path, a decarb sample shall be prepared for LD clarity review.

After decarb sample review:

- A picture of the sample/product shall be uploaded to the designated recordkeeping system.

- The THCa Path shall then be considered complete.

## 5.5 HTE Path

If material is assigned to the HTE Path, the following steps shall be completed:

- HTE shall be placed in the landing zone.

- HTE shall off-gas for 48 hours.

- Following off-gassing, the HTE shall be submitted for testing.

- Test results shall be reviewed to determine whether the HTE is clean.

### 5.5.1 If HTE is Not Clean

If the HTE does not pass cleanliness review, the material shall be moved to the cage upstairs and held for terp stripping/CDTs or other approved alternate handling.

### 5.5.2 If HTE is Clean

If the HTE passes cleanliness review, it shall proceed to terp tube refinement.

After refinement, the material shall be evaluated to determine whether the oil is:

- darker,

- thicker, or

- harder to filter.

#### A. If Yes

If the oil is darker, thicker, or harder to filter, the material shall be processed through the Prescott machine for terp strip and decarb for HP.

After Prescott processing:

- The material shall be sent for testing.

- Potency results shall be reviewed.

If potency is low, the material shall be held for HP base oil.
If potency is high, the material shall be held to be made into distillate.

#### B. If No

If the oil does not require Prescott processing, the material shall be designated as Clean HTE and moved to the menu-ready path.

The HTE terp profile shall then be reviewed against the internal standard for Liquid Loud.

- If the terp profile meets Liquid Loud standards, approximately 2 kg of the strain shall be held for Liquid Loud processing, and the remaining material shall move to the Gold Drop production queue.

- If the terp profile does not meet Liquid Loud standards, the material shall move directly to the Gold Drop production queue.

Once the appropriate routing is complete, the HTE Path is complete.

## 6. Documentation Requirements

The following records shall be completed and maintained for each batch, as applicable:

- Extraction run selection

- Off-gas start and end dates

- Stir logs for pot pour runs

- Transfer to post-processing lab

- Centrifuge completion record

- THCa and HTE output weights

- Oven processing record

- Milling record

- Product path decision

- Decarb sample and uploaded photo

- HTE off-gas record

- Testing submissions and results

- Refinement or Prescott processing records

- Final disposition and queue assignment

All final weights, testing outcomes, and routing decisions shall be recorded in the applicable batch production record and inventory tracking system.

## 7. Quality Control

Testing and internal review shall be completed at all required decision points before material is released into the next stage of processing or production. No material shall be moved forward without proper documentation and approval where required.

## 8. Deviations

Any deviation from this SOP must be documented, reviewed, and approved by management and/or quality personnel before release of affected material.

## 9. Related Documents

- Batch Production Record

- Testing Log

- Equipment Cleaning Logs

- Inventory Transfer Logs

- Product Quality Review Records